Final-Part B

四機三甲 3A511021 朱庭宏

DH Table:

| i | θ | d | a | α |
| --- | --- | --- | --- | --- |
| 1 | θ1 | d1=10 | 0 | 90° |
| 2 | θ2=θ2a+90 | 0 | a2=6.7 | 0 |
| 3 | θ3 | 0 | a3=12 | 0 |

Hardware:

AX-12 motor

- Weight : 53.5g (AX-12/AX-12+), 54.6g (AX-12A)

- Dimension : 32mm * 50mm * 40mm

- Resolution : 0.29°

- Gear Reduction Ratio :  254 : 1

- Stall Torque : 1.5N.m (at 12.0V, 1.5A)

- No load speed : 59rpm (at 12V)

- Running Degree

§       0° ~ 300°

§       Endless Turn

- Running Temperature : -5℃ ~ +70℃

- Voltage : 9  ~ 12V (Recommended Voltage 11.1V)

- Command Signal : Digital Packet

- Protocol Type : Half duplex Asynchronous Serial Communication (8bit,1stop,No Parity)

- Link (Physical) : TTL Level Multi Drop (daisy chain type Connector)

- ID : 254 ID (0~253)

- Communication Speed : 7343bps ~ 1 Mbps

- Feedback : Position, Temperature, Load, Input Voltage, etc.

- Material : Engineering Plastic

Parameters:

| Area | Address (Hexadecimal) | Name | Description | Access | Initial Value (Hexadecimal) |
| --- | --- | --- | --- | --- | --- |
| E E P R O M | 0 (0X00) | Model Number(L) | Lowest byte of model number | R | 12 (0X0C) |
|  | 1 (0X01) | Model Number(H) | Highest byte of model number | R | 0 (0X00) |
|  | 2 (0X02) | Version of Firmware | Information on the version of firmware | R | - |
|  | 3 (0X03) | ID | ID of Dynamixel | RW | 1 (0X01) |
|  | 4 (0X04) | Baud Rate | Baud Rate of Dynamixel | RW | 1 (0X01) |
|  | 5 (0X05) | Return Delay Time | Return Delay Time | RW | 250 (0XFA) |
|  | 6 (0X06) | CW Angle Limit(L) | Lowest byte of clockwise Angle Limit | RW | 0 (0X00) |
|  | 7 (0X07) | CW Angle Limit(H) | Highest byte of clockwise Angle Limit | RW | 0 (0X00) |
|  | 8 (0X08) | CCW Angle Limit(L) | Lowest byte of counterclockwise Angle Limit | RW | 255 (0XFF) |
|  | 9 (0X09) | CCW Angle Limit(H) | Highest byte of counterclockwise Angle Limit | RW | 3 (0X03) |
|  | 11 (0X0B) | the Highest Limit Temperature | Internal Limit Temperature | RW | 70 (0X46) |
|  | 12 (0X0C) | the Lowest Limit Voltage | Lowest Limit Voltage | RW | 60 (0X3C) |
|  | 13 (0X0D) | the Highest Limit Voltage | Highest Limit Voltage | RW | 140 (0XBE) |
|  | 14 (0X0E) | Max Torque(L) | Lowest byte of Max. Torque | RW | 255 (0XFF) |
|  | 15 (0X0F) | Max Torque(H) | Highest byte of Max. Torque | RW | 3 (0X03) |
|  | 16 (0X10) | Status Return Level | Status Return Level | RW | 2 (0X02) |
|  | 17 (0X11) | Alarm LED | LED for Alarm | RW | 36(0x24) |
|  | 18 (0X12) | Alarm Shutdown | Shutdown for Alarm | RW | 36(0x24) |
| R A M | 24 (0X18) | Torque Enable | Torque On/Off | RW | 0 (0X00) |
|  | 25 (0X19) | LED | LED On/Off | RW | 0 (0X00) |
|  | 26 (0X1A) | CW Compliance Margin | CW Compliance margin | RW | 1 (0X01) |
|  | 27 (0X1B) | CCW Compliance Margin | CCW Compliance margin | RW | 1 (0X01) |
|  | 28 (0X1C) | CW Compliance Slope | CW Compliance slope | RW | 32 (0X20) |
|  | 29 (0X1D) | CCW Compliance Slope | CCW Compliance slope | RW | 32 (0X20) |
|  | 30 (0X1E) | Goal Position(L) | Lowest byte of Goal Position | RW | - |
|  | 31 (0X1F) | Goal Position(H) | Highest byte of Goal Position | RW | - |
|  | 32 (0X20) | Moving Speed(L) | Lowest byte of Moving Speed (Moving Velocity) | RW | - |
|  | 33 (0X21) | Moving Speed(H) | Highest byte of Moving Speed (Moving Velocity) | RW | - |
|  | 34 (0X22) | Torque Limit(L) | Lowest byte of Torque Limit (Goal Torque) | RW | ADD14 |
|  | 35 (0X23) | Torque Limit(H) | Highest byte of Torque Limit (Goal Torque) | RW | ADD15 |
|  | 36 (0X24) | Present Position(L) | Lowest byte of Current Position (Present Velocity) | R | - |
|  | 37 (0X25) | Present Position(H) | Highest byte of Current Position (Present Velocity) | R | - |
|  | 38 (0X26) | Present Speed(L) | Lowest byte of Current Speed | R | - |
|  | 39 (0X27) | Present Speed(H) | Highest byte of Current Speed | R | - |
|  | 40 (0X28) | Present Load(L) | Lowest byte of Current Load | R | - |
|  | 41 (0X29) | Present Load(H) | Highest byte of Current Load | R | - |
|  | 42 (0X2A) | Present Voltage | Current Voltage | R | - |
|  | 43 (0X2B) | Present Temperature | Current Temperature | R | - |
|  | 44 (0X2C) | Registered | Means if Instruction is registered | R | 0 (0X00) |
|  | 46 (0X2E) | Moving | Means if there is any movement | R | 0 (0X00) |
|  | 47 (0X2F) | Lock | Locking EEPROM | RW | 0 (0X00) |
|  | 48 (0X30) | Punch(L) | Lowest byte of Punch | RW | 32 (0X20) |
|  | 49 (0X31) | Punch(H) | Highest byte of Punch | RW | 0 (0X00) |

DOF:3

Motor Parameters Setup:

- ID:1 to 4

- Baud Rate: 1000000 bps

- GoalPosition(L):512(150°) [for center position]

U2D2 Spec:

U2D2 Layout:

Connect to PC via U2D2:

Connection:

Software:

Roboplus>>Dynamixal Wizard

LabVIEW

程式

Joint Space

Cartesian Space

P2P(joint)

P2P(Cartesian)

Duplicate

正向運動學

逆向運動學

A to B N Points (分點)

程式改良

1.新增復歸模式

2.複製模式新增AB互換，使其可回到上一點

3.於卡式空間須按下GO才會移動到目標點

4.顯示當前位置

Package

完成圖